2.2 Airborne Doppler remote sensing

| Scanning method | Vertical about the aircraft’s longitudinal axis; fore/aft simultaneous sweep methodology |
| --- | --- |
| wavelength | 3.2 cm |
| beamwidth | 1.8 deg |
| polarization | Liner horizontal |
| sldelobes | -35 dB |
| gain | 39 dB |
| Antenna rotation rate | Variable: 5-144 deg/s |
| Fore/aft tilt | +/-15-19 deg, depending on frequency |
| Pulse repetition frequency | Variable 2000-5000/sec |
| Dual PRF ratios | variable |
| Pulses averaged per radial | variable |
| Pulse width | .25-3 msec |
| Rotation sampling rate | .075-2 deg |
| Peak transmitter power | 35-40 kW |
| Unambiguous range | 20-90 km |
| Unambiguous radial velocity | 13-110 m/s |
| Number of transmitted frequencies | 4 |
| Along-track beam spacing | ~300 m |
| Range resolution | 37.5 m – 1200 m |
| Table 2.4 Characteristics of ELDORA (from RAINEX operations plan) |  |

The characteristics of the NRL P-3 ELDORA x-band radar are listed in Table 2.4. ELDORA transmits up to four frequencies during a sampling period to increase the number of independent samples in each range sample to allow for a faster antenna scan rate (to ~144° s-1) without sacrificing measurement accuracy. The system employs a multiple PRF (pulse repetition frequency) scheme to extend their respective radial velocity Nyquist intervals to > 50 m s-1, greatly easing the work required to dealias (or unfold) radial velocity data sets.

The ELDORA Doppler radar antennas are mounted in the tail of the aircraft. They utilize the fore/aft scanning technique to sweep out a three-dimensional volume during the aircraft’s flight track. ELDORA transmits both fore and aft looking beams simultaneously. At intersection points of the fore and aft beams, a horizontal wind estimate can be made. The horizontal data spacing of those intersection points depends on the antenna rotation rate and the ground speed of the aircraft. For typical values of ground speed, the differences in antenna rotation rate lead to a ~300 m horizontal data spacing.

2.3 Dropsondes

| NRL P-3 | 300 |
| --- | --- |
| WC-130J | 750 |
| DLR Falcon | 300 |
| Table 2.5 Dropsondes allocated to each of the three T-PARC aircraft |  |

A plethora of dropsondes will be available in T-PARC (Table 2,5), and will be launched from each of the three aircraft, but mainly from the Falcon and C-130J.  All participating

aircraft have GPS dropsonde capability, and drops will be performed at varying frequencies, in accord with the flight plans in Sec. 3. All aircraft will be using the latest versions of dropsondes manufactured by Vaisala (RD90 and/or RD93). Table 2.6 lists general specifications of the dropsonde expendable package. All facilities will be requested to provide high resolution raw data to T-PARC. A combination of facility and EOL data quality checks will be performed prior to data archival.

| Size | 7 cm diameter, 41 cm long |
| --- | --- |
| Weight | <400 g |
| Max Deployment Airspeed | 125 m/s indicated air speed |
| Descent speed | ~11 m/s at sea level (adjustable with chute type) |
| Descent time | ~15 min. from 14 km; 8 min from 7.5 km |
| Transmitter | RF, range 400-406 MHz |
| Telemetry range | 325 km |
| Battery | 2 h operating life, Lithium |
| Receiver System | AVAPS multi-dropsonde unit |
| Pressure | Vaisala BAROCAP, accuracy 0.4 hPa |
| Temperature | Vaisala THERMOCAP, accuracy 0.2ºC |
| Relative humidity | Vaisala H-HUMIDAD thin film capacitive bead. Accuracy 2% RH |
| Horizontal winds | GPS derived, accuracy 0.5 m/s |
| Table 2.6 Dropsonde characteristics (from RAINEX operations plan) |  |