12/08/09

Aircraft Research Instrumentation

GV - Basic RAF ADELE/SPRITE Configuration

Smith / Nielsen

RAF-Supplied Instrumentation:

I.  Airborne Data System.

A. Acquisition.  GV ADS3.

NCAR Distributed Sampling Modules (9 units), one in

each of 6 wing pylons, 1 in radome, and 2 in the cabin.

B. Cabin Displays(3):

Dell Latitude Pentium MMX-266 Laptops (3)

Mission Coordinator Station

II. Aircraft Position, Velocity and Attitude

A. Honeywell Laseref IV SM Inertial Reference Systems (2).

(Name, Name_IRS2).

B. Garmin GPS16 Global Positioning System. (GGName_GMN).

C. NovAtel Model OEM 4 L1/L2 Differential GPS High Accuracy

Positioning System (GGName)

D. Avionics GPS (Name_G)

III.Static Pressures.

A. Paroscientific Model 1000 Digital Pressure Transducer -

Fuselage Port (PSF).

B. Avionics Sensor - Corrected (PS_A).

IV. Dynamic Pressures.

A. Honeywell PPT (0-5 PSI) Absolute Pressure Transducer – Radome

(QCR).

B. Honeywell PPT (0-5 PSI) Absolute Pressure Transducer  – Nose

Cone Pitot (QCF).

C. Avionics Sensor – Corrected (QC_A).

V.  Temperatures.

A. Rosemount Type 102 Non-deiced Sensor -- Rosemount Model

510BF Amplifier - Port Fuselage Mount (TTFR).

B. HARCO De-iced Dual Temperature Sensor – Starboard Fuselage

Mount (TTHR1, TTHR2).

C. Avionics Heated Rosemount Temperature Sensor – Port Fuselage

Mount (TT_A, AT_A).

VI. Flow Angle Sensors, Radome.

A. Attack – Honeywell PPT (+- 1 PSI) Differential Pressure

Transducer (ADIFR).

B. Sideslip – Honeywell PPT (+- 1 PSI) Differential

Pressure Transducer (BDIFR).

GV RAF ADELE

Page 2

VII. Dew Point and Humidity.

A. Buck Research, Model 1011C Dew Point Hygrometer –

Port Fuselage Mount (DPLS).

B. Buck Research, Model 1011C Dew Point Hygrometer - 	    	   	   Starboard Fuselage Mount (DPRS).

VIII. User Supplied Instrumentation

A. Phantom-7 High Speed Imager (2 units) – Port side Cabin.

B. ADELE – Gamma Ray Detectors

IX. Wing Stores (A,B,C,D,I for RAF test, only)

A. All Weather Wing Gust Pod. Wing Position - LOI

B. LAMS Laser Air Speed. Wing Position - LOO

C. HOLODEC-II. Wing Position - ROI

D. Microwave Temperature Profiler. Wing Position - LCO

- RAF – DMT Cloud Droplet Probe

Size Distribution, 3 – 45 um. Wing Position - RCO

F. RAF – Modified PMS 2D-C Optical array Probe

Size Distribution, 25 to 800 um. Wing Position - RIO

G. RAF – DMT UHSAS Aerosol Probe.  Wing Position - LIO

Size Distribution, 0.1 – 3 um.

H. Rosemount Model 871 Icing Rate Detector.  Wing Position – RO

I. RAF – Small Ice Detector (SID-2). Wing Position - ROO

X. Photography.

A. Point Grey Model Flea Digital Video Camera.  Forward facing

in wing pylon. (1 sec images recorded).