8/30/2013

Aircraft Research Instrumentation

GV - Basic RAF SPRITE-II Configuration

Nielson et al

RAF-Supplied Instrumentation:

I.  Airborne Data System.

A. Acquisition.  GV ADS3.

NCAR Distributed Sampling Modules (3 units), 1 in radome,

and 2 in the cabin.

B. Mission Scientist Display:

Dell Latitude Pentium MMX-266 Laptop

Watec Camera Displays

C. Mission Coordinator Display:

D. Phantom Camera Display /w trigger

II. Aircraft Position, Velocity and Attitude

A. Honeywell Laseref IV SM Inertial Reference Systems (2).

(Name, Name_IRS2).

B. Garmin GPS16 Global Positioning System. (GGName_GMN).

C. NovAtel Model OEM 4 L1/L2 Differential GPS High Accuracy

Positioning System (GGName)

D. Avionics GPS (Name_G)

III. Static Pressures.

A. Paroscientific Model 1000 Digital Pressure Transducer -

Fuselage Port (PSF).

B. Avionics Sensor - Corrected (PS_A).

IV. Dynamic Pressures.

A. Honeywell PPT (0-5 PSI) Absolute Pressure Transducer – Radome

(QCR).

B. Honeywell PPT (0-5 PSI) Absolute Pressure Transducer  – Nose

Cone Pitot (QCF).

C. Avionics Sensor – Corrected (QC_A).

V.  Temperatures.

A. Rosemount Type 102 Sensor -- Rosemount Model

510BF Amplifier - Port Fuselage Mount (TTRL)).

B. HARCO De-iced Dual Temperature Sensor – Starboard Fuselage

Mount (TTHR1, TTHR2).

C. Avionics Heated Rosemount Temperature Sensor – Port Fuselage

Mount (TT_A, AT_A).

VI. Flow Angle Sensors, Radome.

A. Attack – Honeywell PPT (+- 1 PSI) Differential Pressure

Transducer (ADIFR).

B. Sideslip – Honeywell PPT (+- 1 PSI) Differential

Pressure Transducer (BDIFR).

GV RAF SPRITE-II

Page 2

VII. Dew Point and Humidity.

A. Buck Research, Model 1011C Dew Point Hygrometer –

Port Fuselage Mount (DP_DPL).

B. Buck Research, Model 1011C Dew Point Hygrometer - 	    	   	   	   Starboard Fuselage Mount (DP_DPR).

XI. User Supplied Instruments

- HS Phantom Camera #1

- HS Phantom Camera #2

- Watec Camera #1

- Watec Camera #2

- Watec Camera #3

- ANDOR Camera