Adele/Sprite

RF01

TECH:Irwin

After waiting around 7 hours, we took off at 5 pm to fly some

thunderstorms near the Tampa Bay area.  Flew for approx. 3 hours,

(finishing up now), and had everything seemingly work well except the

nose DSM.  We think it's the timing board, as twice the DSM went

"south" with it's time and once with it's date also, stopping all data

collection from that DSM.  The fix both times was resetting the C/B for

that DSM on the nose, came back both times.

22:12 and 22:57 were the times of the dropouts, the first was fixed at

22:21 counting reboot time and the last was back up at 23:15.  Was up

talking to pilots so didn't see it happen right away.

We are parked outside, and sometimes have to deal with rain and

lightning, we've developed our own set of guidelines with this, which

twice as made us shut the airplane down and get everyone off the

airplane into the building, due to lightning getting close.

So, with the almost end of TF-01, Kurt we request you send us another

timing board, as I suspect we'll be changing out the nose DSM one with

our spare tomorrow.

Bill