START08

RF03

TECH: Munnerlyn

April 24, 2008

1456Z		Wheels up

1856Z		Gaps in Data reported in Aeros

Informed ground by chat, fixed by remote login of SE

2051Z		Quant WCN at R5 flashes message water in drain line

Informed ground by chat, diagnosis and directions to fix relayed by chat

2054Z		DSM313 stopped reported by PI, cycled power at DSM, function restored

2101Z		DSM312 stopped, cycled power at DSM, function restored

Blank computer display for MTP, checked KVM at R5, asked by PI to look for lights on computer, computer verified as on. Mouse stopped functioning while diagnosing, wiggled mouse connection to KVM (known problem), mouse came back. This problem was mainly worked by PI and ground support. MTP computer did not regain function for rest of flight.

PLWC shows jump in output in Aeros with spikes. Returned to normal with no intervention.