RF07

TECH:  DEA

Date:  5/4/2008

Takeoff:  1004

Landed:  1535

0955 – DSMs appeared to be locked up looking at the ADS Interface page.  I reset the DSMs which did not solve the problem…  Then I cycled the ADS interface page and this fixed the problem.  The problem was the ADS Interface page.  Just needed to be cycled.

1019 – The Total Temps were hosed up (which later we found out to be a temperature issue, cycling power prior to take off caused issues) so I reset the NOSE/MPDB DSM switch.  Total temps still In-Op.  After takeoff, getting some airflow in the nose, the Nose DSM (A/D card cooled off enough to where cycling the NOSE/MPDB DSM switch again got the Total Temps functioning properly at 1038.

1130 – CO1, DSM 313 power cycled.