RF07

TECH: Cowan

10/31/08

RR arica to at 0600Z for 9 hours.

2DC 10um inop with a bad laser.

DPB is inop no lights and no display now.  It comes up but will not stay up.  Will replace after the flight.

After moving to the fuel pits found that the cockpit display did not have a good network connection.  Rebooted and it came back up.

0500 moved to Fuel pits.

0557 Taxi

0603 Takeoff

0615, cockpit laptop has access to srt_raw data base.  It should not since it is not used onboard. This was the only laptop that had access to that data base.

0621 DSM Forward #2 reporting NOPPS in read.

0623 Forward DSM now okay on the PPS.

0729 Track plot not working on Aeros.  Not getting any latest Lat, Lon, Alt. All are clustered in the center of the page. POST flight note, Chris said that if I had restarted the areos sessions it would have worked afterwards.

0800 TTRR has a large singles spike, both nose temps but not the wing. POST flight note: This was seen in all analogs for DSM 320 and 321. It was a 5 second data gap or shift.

0814 XUVI voltage -32767 because we are high and drive and V is 11+.

0820 XUVI is back reading correctly.

0908 going through cloud and found SPP-100 not talking.  No laser ref found.  Will reboot DSM at end of cloud run.

0917 Looks like SPP-100 is back with a good laser ref. Lost at 0851 got back at 0915.  I did nothing to get it started again.

0924 I do not think that the SPP-100 is correct even though we are getting laser ref. Wed will reboot at the end of this leg per PI request.

0940 will reboot at the end of the run.

0948 commencing RW tip probe reboot and DSM 324 reboot should take 2 minutes to recover.

0950 reset worked, probe looks much better.

1028 More Conc on 2DC than any other probe. The 2dc has Conc in L/min while the others are in cm3/min.  That is the difference.

1305 noticing GPS dropout in NS and EW and the track and the status.

1457 Landing.