RF11

TECH: Cowan

1/30/09

Flight from Costa Rica to BJC.

VCSEL not functioning. Server issues form last flight seem to have been fixed.

0957L Taxi

1000L Takeoff

1800 Found LW DSM stopped, it stopped at 1754, rebooting from console.

1919 IRS wind speeds (IWS and IWS_IRS2) have dropped compared to others (WS).  Noisy incident about 1912. Not sure why this is at this point. WD for both IRS and ours are the same.

2011 WS was in excess of 65 ms, we are wondering if this is a hard coding limit and that we have wrapped around since the wind went from 65 ms to -65 ms.

2115 Need increase the limit to at lease 80 ms as that was the highest wind we saw.

2031 Rebooting LW DSM due to analog issue. I think the PLWC was not displaying data.  Analog card on DSM status page said the analog card was not active, that is why it was rebooted. Original time of drop was 2007.

Note: The iridium control in the cockpit has a blinking light and when it is used has a worbling tone. The handset does not have the tone and works fine.

2106 LW DSM no analog data, having SE look at it.

2120 Rebooted LWO DSM.

2132 LW DSM analog inactive again.  Will reboot before landing.

2155 Rebooted LW DSM

2207 Landing