RF12

TECH: Ta

Take off : 9:52 AM

Right after taking off, Water from aircraft AC. Over head system was dump in to ADS computer rack heavily .Luckily no water cause damage was found and ADS keep functional for entire flight.

Reference Voltages RW Inboard probe was hung at 13:24 .Reboot DSM & Probe.

Horita Titler  has a lot of noises and it didn’t updating .Problem found: BNC connector on the back was came loses correct problem and everything went well.

DPT &DPB values from Ascii list were not agreed with.Dewpoint indicator values. Jon suggests rebalance dew pointers (A quick analyzer problem after flight was showed: Nothing wrong with sensors it just only computer display).

Reset probe SPP200 at: 20:54 GMT due to It was hang up

Again Iridium for chat system was down :9 time for entire flight.

Landed:6:43PM