RF13

TECH: Cowan

Pre-Flight – Found Variable A2DTemAft2 this goes to a card that is not in the DSM and is no good.

No C-Migets – We could not talk to the unit with the program.

Block Out 1256

Take Off 1300

1314 FWDDSM2 Had no PPS but got it while I was watching.

1315 Offset between the dew point sensors of about 2 C. They are tracking.  They can back together at 1318.

1824 1.5 degree offset on TT’s and it has been there for a little while.

2010 Cindy rebooted CVI DSM, no analog data.

2157 Landing