RF14

TECH: G.B.

6/18/08

Takeoff:  17:04

19:09 – 19:16 -  SATCOM Dropout

20:40 – DSM 312 Stopped

20:48 – DSM 312 Back up

21:03 – Lost Data from 2DC Probe

21:07 – QCR is sluggish and not tracking with QCF

21:17 - Cycled power on LW to try and revive 2DC

21:19 – LWDSM Backup

21:24 – LWDSM power cycle because of booting of 2DC

21:26 – LWDSM Backup

23:22 – Left Dewpoint is way off

23:48 – Left Dewpoint recovered

Landing:  00:10

- Large spikes in temps from 21:15 to the end of flight.  Probably 8-10 of them.  After flight the High Rate data still appears to be good.

- After flight found water in QCR line.

- After flight found 2DC to be non-functional.  Is currently being repaired.

Many of the problems that occurred on this flight happened during a period when we went over part of a thunderstorm with a fair amount of turbulence.

Also during the same time in the flight there was a “hot” smell that seemed to be associated with the MEDUSA pump.  The breaker was turned off.  After the flight the possibility was discussed of the smell coming from the air packs of the GV due to the turbulence at the time.