RF17

TECH: Cowan

Date: 6/26/08

RR out of Jeffco with inter-comparison with DC8.

1640 Block out

1649 Takeoff

1657 Lost chat during the climb out, no sure why.

1725 Noticed that the DSM times on the web page were not updating. They had not been updating since about 1630.  Restarted Mozilla and the times started to update.  Data was being recorded and that was verified by the fact that new data was being displayed at areos.

2113 Keyboard locked up.  The enter key was stuck and opened a large number of chat windows.  This caused the ADS to be banned from the chat channels.  After fixing the keyboard (unplug/replug and whapping the enter key) I had to get Mark Bradford to unban ADS for the chat channels.

2236 Turned on and off both dew point sensors because the service mirror lights came on  during the low approach to Clarksville.

2238 Turned on and off Right Dew Point sensor because it was giving service mirror light again.

0001 Large spike in PLWC for no apparent reason.

0010 Have not needed to reboot the Left DSM (312) this flight.  It was rebooted during preflight and maybe that fixed the issue for this flight.

0052 Landing

0055 Block in