TF02

TECH: Cowan

Date 4/14/08

Preflight – Had an issue with laptops and DHCP.  Need to not turn on laptops until server is completely up.

Block Out - 1912

Take Off – 1915

1923 Large spike in PLWC up to almost 80W. No other spike an any other probes.

1937

Left A Bus

400 A Phase 19 A

400 B Phase 13.5 A

400 C Phase 15.9 A

400 all Phase 115 Volts

400 AI

All 0 with 115V

60Hz  116 V  22 A

28Volts AI  29.1 V 10 A

Right Side B Bus

400 A Phase 10.1 A

400 B Phase 15.5 A

400 C Phase 10.3 A

400 all Phase 115 Volts

400 AI

A 0

B 4A

C 0

All 115V

60Hz 115 V 18 A

All the amp meters seem to be bouncing a lot. Left A is the worse with a 5A swing.

1947 A/D temp on LWO DSM went to 508 C. It is a wrap around issue it went to 0 and then went to 500C.

2006 No indication from the 03 instruments, all are reporting -32767. (Found out after flight that 2 of the instruments report ant once every 10 seconds and will show -32767 for the 9 seconds they are not reporting.  Look at DSM status to see if they are connected.)

1950 should see good CDP data and maybe 2DC data.

2123 Another big PLWC spike to 70W.  Aircraft descending at that time.

2143 Landing.